ToxGenie Multi-Group Comparison Report

Report Date: 2026-06-24 15:55:25

| Study Title: | Reproduction Test |
| --- | --- |

| Analysis Method: | Dunnett's Multiple Comparison Test (via Log transformation) for Avg Dry Weight |
| --- | --- |

| Study No.: | Test-135 | Test Material: | Test Chemicals |
| --- | --- | --- | --- |
| Exposure Time: | 24-hours | Test Medium: | Elendt M4 |

Table 1. Raw Data Summary for Avg Dry Weight

(Unit: mg/L)

| Dose | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| Control | 1.2900 | 1.3200 | 1.5900 | 1.2700 |
| 6.25 | 1.2700 | 1.0000 | 1.0800 | 0.9700 |
| 12.5 | 1.3200 | 1.3700 | 1.3500 | 1.3400 |
| 25 | 1.2900 | 1.3300 | 1.2000 | 1.1700 |
| 50 | 0.7800 | 0.7000 | 0.6600 | 0.7700 |

Step 1. Outlier Detection (Data QA/QC) Analysis

Table 2. Identified Statistical Outliers

| Dose | Replicate No. | Observed Value | Normal Range (1.5 IQR) |
| --- | --- | --- | --- |
| Control | 3 | 1.5900 | [1.1313, 1.5413] |

Note: Outliers were identified using Tukey's fences (Values outside Q1 - 1.5*IQR or Q3 + 1.5*IQR).

These values are flagged for Quality Assurance purposes but are typically retained to reflect true biological variance, unless experimental error is proven.

Step 2. Descriptive Statistics

Table 3. Descriptive Statistics for Avg Dry Weight

(Unit: mg/L)

| Dose | N | Mean | SD | SE | CV (%) | Median |
| --- | --- | --- | --- | --- | --- | --- |
| Control | 4 | 1.36750 | 0.14975 | 0.07487 | 10.95062 | 1.30500 |
| 6.25 | 4 | 1.08000 | 0.13491 | 0.06745 | 12.49142 | 1.04000 |
| 12.5 | 4 | 1.34500 | 0.02082 | 0.01041 | 1.54771 | 1.34500 |
| 25 | 4 | 1.24750 | 0.07500 | 0.03750 | 6.01202 | 1.24500 |
| 50 | 4 | 0.72750 | 0.05737 | 0.02869 | 7.88633 | 0.73500 |

Abbreviations & Explanations:

- N: Number of replicates, Mean: Average response value.

- SD: Standard Deviation, SE: Standard Error, CV (%): Coefficient of Variation.

Table 4. Shapiro-Wilk's test for normality on the Avg Dry Weight

| Dose | Statistic | P-value | Result |
| --- | --- | --- | --- |
| Control | 0.7519 | 0.0403 | Reject Normal Distribution |
| 6.25 | 0.8823 | 0.3486 | Normal Distribution |
| 12.5 | 0.9984 | 0.9951 | Normal Distribution |
| 25 | 0.9249 | 0.5649 | Normal Distribution |
| 50 | 0.8970 | 0.4162 | Normal Distribution |

P>0.05 indicates Normal Distribution.
Final decision: The Avg Dry Weight data does not follow a normal distribution.

Table 5. Levene's test for homogeneity on the Avg Dry Weight

| Statistic | P-value | Result |
| --- | --- | --- |
| 0.78026 | 0.55523 | Equal Variances |

P>0.05 indicates Equal Variances.
Final decision: The Avg Dry Weight data shows equal variances.

Table 6. Transformed Data Summary for Avg Dry Weight (Log transformation)

| Dose | 1 | 2 | 3 | 4 | Mean | SD |
| --- | --- | --- | --- | --- | --- | --- |
| Control | 0.2546 | 0.2776 | 0.4637 | 0.2390 | 0.2058 | 0.1788 |
| 6.25 | 0.2390 | 0.0000 | 0.0770 | -0.0305 | 0.3530 | 0.7314 |
| 12.5 | 0.2776 | 0.3148 | 0.3001 | 0.2927 | 0.6185 | 0.9419 |
| 25 | 0.2546 | 0.2852 | 0.1823 | 0.1570 | 0.6830 | 1.2463 |
| 50 | -0.2485 | -0.3567 | -0.4155 | -0.2614 | 0.4383 | 1.7077 |

Note: Data were transformed using the log method prior to statistical analysis.

Table 7. Shapiro-Wilk's test for normality on the transformed Avg Dry Weight.

| Dose | Statistic | P-value | Result |
| --- | --- | --- | --- |
| Control | 0.76414 | 0.05192 | Normal Distribution |
| 6.25 | 0.89921 | 0.42713 | Normal Distribution |
| 12.5 | 0.99836 | 0.99492 | Normal Distribution |
| 25 | 0.92503 | 0.56550 | Normal Distribution |
| 50 | 0.89749 | 0.41870 | Normal Distribution |

P>0.05 indicates Normal Distribution.
Final decision: The data follows a normal distribution.

Table 8. Levene's test for homogeneity on the transformed Avg Dry Weight.

| Statistic | P-value | Result |
| --- | --- | --- |
| 1.01300 | 0.43181 | Equal Variances |

P>0.05 indicates Equal Variances.
Final decision: The data shows equal variances.

Statistical Analysis Procedure & Decision Tree

The statistical analysis for 'Avg Dry Weight' was conducted based on the following decision steps:

1. Assumption Verification (Normality & Homogeneity):

Since the raw data did not meet the assumption(s) of Normality, data transformation was attempted to stabilize variance and normalize the distribution.

2. Data Transformation Strategy:

A Log transformation was selected and applied.

Logarithmic transformation reduces skewness in positive data and stabilizes variance when it increases with the mean. It was chosen because it yielded the highest combined score for Normality and Homogeneity among all candidates.

The transformed data successfully met the required statistical assumptions (Normality and Homogeneity of Variance). Thus, Parametric Tests were performed on the transformed data.

3. Final Selected Test Method:

Because after applying data transformation, the transformed data successfully met the required statistical assumptions (Normality and Homogeneity of Variance), the Dunnett Test (Parametric) was selected as the post-hoc test.

Prior to this post-hoc analysis, an Omnibus Test (One-Way ANOVA) was performed to confirm if any statistically significant differences exist among the dose groups (p < 0.05).

The selected method is appropriate for comparing multiple treatment groups against a single control group to determine the NOEC/LOEC (No Observed Effect Concentration/Lowest Observed Effect Concentration) or NOAEL/LOAEL (No Observed Adverse Effect Level/Lowest Observed Adverse Effect Level).

Table 9. One-Way ANOVA on the transformed Avg Dry Weight data.

| F-statistic | P-value | Result |
| --- | --- | --- |
| 38.2808 | 0.0000 | Significant differences among doses |

P<0.05 indicates a statistically significant difference among groups.

Table 10. Post Hoc Analytical Method: Dunnett's Test on the transformed Avg Dry Weight data.

| Control | Dose | P-value | Result |
| --- | --- | --- | --- |
| Control | 6.25 | 0.0043 | Significant difference from Control |
| Control | 12.5 | 0.9985 | No significant difference from Control |
| Control | 25 | 0.4043 | No significant difference from Control |
| Control | 50 | 0.0000 | Significant difference from Control |

P<0.05 indicates a statistically significant difference from the Control.

Figure 1. Dose-response trend for Avg Dry Weight. Bars represent the mean and Standard Deviation (SD).

|  | Analyst: | ToxGenie |
| --- | --- | --- |